|  | Carlson Tool and Machine Co. Standard Jaw Staple Attachment Sizes For Brush Making Set Equipment |
| --- | --- |

Small Hole Equipment: 	Model #OT-22, 21, 22 cam, 44 cam, 11 and OC-44

Hole Size	Attachments	  Wire Diameter	Straight Staple	   Staple Legs

| #54 - #47 | 3/64” | 26 ga./.018 | 5/16” | 1/8” |
| --- | --- | --- | --- | --- |
| #49 - #45 | Sp 1/16”T | 26 ga./.018 | 5/16” | 1/8” |
| #44 - #31 | 1/16” | 25 ga./.020 | 3/8” | 5/32” |
| #32 - 9/64” | 5/64” | 22 ga./.028 | 3/8” with 3/32 anvil | 5/32” |
| 1/8” – 5/32” | 3/32” | 22 ga./.028 | 9/16” | 3/16” |
| 11/64” – 3/16” | 1/8” | 20 ga./.034 | 5/8” | 1/4” |
| 13/64” – 1/4” | Sp 5/32” | 19 ga./.041 | 3/4” | 5/16” |

Large Hole Equipment:  Models #OT-17, OT-14,10,10L14,14L,16,16L,17,17L & 41

(Note: This covers three staple jaw tool stroke lengths)

Hole Size	Attachments	  Wire Diameter	Straight Staple	   Staple Legs

| 1/8”-5/32” | 3/32” | 22 ga./.0286 | 5/8” | 1/4” |
| --- | --- | --- | --- | --- |
| 5/32”-13/64” | 1/8” | 20 ga./.0345 | 3/4” | 5/16” |
| 7/32”-17/64” | 5/32” | 19 ga./.041 | 7/8” | 3/8” |
| 9/32”-5/16’ | 3/16” | 18 ga./.0475 | 1” | 7/16” |
| 5/16” – 25/64” | 1/4” | 16 ga./.065 | 1 1/4” | 1/2” |

Extra Long Stroke / Large Hole:   Model #12, 46, 46S, 19, 19T and OT-46

Covers 1/8” Attachment listed above in closed staple pivot jaws.

Covers 5/32” to 1/4” Attachments in fixed style staple jaw tooling as above.

(In larger Attachments the hole size range will overlap)

13/64” – 1/2” holes, 9/32” Tooling using 15 ga. / .072 Brush Staple Wire

Over 1/2” – 5/8”, Special 5/16” Jaw Tooling with 14 ga. /.082 Staple Wire